|  | Simulation of c06_tut01 Date: Wednesday, December 06, 2017 Designer: Solidworks Study name: Tuning Fork Analysis type: Frequency. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 4 Contact Information 5 Mesh information 6 Sensor Details 7 Study Results 8 Conclusion 12. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: c06_tut01 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet5; Solid Body; Mass:0.140075 kg Volume:1.79583e-05 m^3 Density:7800 kg/m^3 Weight:1.37273 N; C:\Users\Sham\Desktop\SWS2018\Input Files\c06_simulation_2016_input\c06_tut01\c06_tut01.SLDPRT Jun 2 10:39:42 2017.

# Study Properties Study name; Tuning Fork. Analysis type; Frequency. Mesh type; Solid Mesh. Number of frequencies; 5. Solver type; FFEPlus. Soft Spring:; Off. Incompatible bonding options; Automatic. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Result folder; SOLIDWORKS document (C:\Users\Sham\Desktop\SWS2018\Input Files\c06_simulation_2016_input\c06_tut01).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; Chrome Stainless Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 1.72339e+08 N/m^2. Tensile strength:; 4.13613e+08 N/m^2. Mass density:; 7800 kg/m^3. Elastic modulus:; 2e+11 N/m^2. Poisson's ratio:; 0.28. Thermal expansion coefficient:; 1.1e-05 /Kelvin.; SolidBody 1(Fillet5)(c06_tut01). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 2.61967 mm. Tolerance; 0.130984 mm. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 13429. Total Elements; 7802. Maximum Aspect Ratio; 17.747. % of elements with Aspect Ratio < 3; 98.9. % of elements with Aspect Ratio > 10; 0.0641. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:05. Computer name:; AVIJEET.

# Sensor Details No Data

# Study Results Name; Type; Min; Max. Amplitude1; AMPRES: Resultant Amplitude Plot for Mode Shape: 1(Value = 126.117 Hz); 0.000e+00 Node: 47; 4.561e+00 Node: 447. c06_tut01-Tuning Fork-Amplitude-Amplitude1. Name; Type; Min; Max. Amplitude2; AMPRES: Resultant Amplitude Plot for Mode Shape: 2(Value = 127.908 Hz); 0.000e+00 Node: 47; 4.643e+00 Node: 235. c06_tut01-Tuning Fork-Amplitude-Amplitude2. Name; Type; Min; Max. Amplitude3; AMPRES: Resultant Amplitude Plot for Mode Shape: 3(Value = 345.83 Hz); 0.000e+00 Node: 47; 4.791e+00 Node: 444. c06_tut01-Tuning Fork-Amplitude-Amplitude3. Name; Type; Min; Max. Amplitude4; AMPRES: Resultant Amplitude Plot for Mode Shape: 4(Value = 649.395 Hz); 0.000e+00 Node: 47; 5.620e+00 Node: 235. c06_tut01-Tuning Fork-Amplitude-Amplitude4. Name; Type; Min; Max. Amplitude5; AMPRES: Resultant Amplitude Plot for Mode Shape: 5(Value = 1047.3 Hz); 0.000e+00 Node: 47; 5.863e+00 Node: 228. c06_tut01-Tuning Fork-Amplitude-Amplitude5. Mode List Frequency Number; Rad/sec; Hertz; Seconds. 1; 792.41; 126.12; 0.0079292. 2; 803.67; 127.91; 0.0078181. 3; 2172.9; 345.83; 0.0028916. 4; 4080.3; 649.39; 0.0015399. 5; 6580.4; 1047.3; 0.00095484. Mass Participation (Normalized) Mode Number; Frequency(Hertz); X direction; Y direction; Z direction. 1; 126.12; 1.1277e-10; 0.72711; 6.4562e-06. 2; 127.91; 2.2271e-10; 6.087e-06; 0.76455. 3; 345.83; 2.2399e-12; 8.1139e-09; 1.8635e-10. 4; 649.39; 0.0088442; 6.7836e-11; 3.8712e-13. 5; 1047.3; 1.5396e-10; 6.3008e-09; 4.8323e-09. Sum X = 0.0088442; Sum Y = 0.72712; Sum Z = 0.76456.

# Conclusion